TruTrak Serial Remote Autopilot Interface

| Revision | Notes | Date |
| --- | --- | --- |
| IR | Created Document | 23-4-2014 |
| Rev 1 | Changed sentences to be proprietary labels and added command, status modes for following ARINC commands instead of serial, added ARINC interface, added in all other existing interfaces that we currently use for full AP control. | 14-5-2014 |
| Rev B | Corrected $PTFSM message to be $PTFSA message. | 6-14-19 |
| Rev C | Corrected message labels, updated message breakdowns to reflect current operation | 8-8-19 |

All communication and remote control of the TruTrak Vizion autopilot can be provided over a single bi-directional RS-232 interface.  There are five sentences that can be sent to the autopilot for this control.  These sentences are:

$PGALV, $PTFSB, $PGRMZ, $GPRMB, $GPRMC.

Additionally, the autopilot sends a return status sentence which is $PGASR.

The RS-232 interface is operated at 9600 baud, no parity, 1 start bit, 8 data bits and 1 stop bit.

The following messages are sent by the EFIS / AHRS:

$PGALV is sent at 5Hz.  This sentence contains the lateral and vertical guidance commands for the autopilot.

$PTFSB is sent as needed.  This sentence is for autopilot mode control.

$PGRMZ is sent at 1Hz.  This sentence contains pressure altitude and barometer correction data for the autopilot. – not currently used

$GPRMC is sent at 1Hz.  This sentence contains basic GPS track, speed, and other information.

$GPRMB is sent at 1Hz.  This sentence contains GPS flight plan information.

The following message is sent by the autopilot:

$PGASR is sent every two seconds or when a change has occurred.

Note1: For basic flight plan following by the autopilot the $GPRMC and $GPRMB sentences must both be present.

Note2: For full autopilot control the remaining sentences shall be sent.  When using full autopilot control, the $GPRMB sentence may be omitted.

$PGALV

The format of this message is:

$PGALV,RollCmd,VSCmd,PitchCmd,Roll,VS,Pitch,IAS*CS<CR><LF>

| Field | Data | Description |
| --- | --- | --- |
| $PGALV | Identifier | Sentence identifier with $ starting character |
| RollCmd | sdddd | Commanded Roll Angle s -(roll left) +(roll right) dddd tenths of degrees |
| VSCmd | sdddd | Commanded Vertical Speed s -(descend) +(climb) dddd feet/min Note: Data field is empty when using pitch guidance. If both VSCmd and PitchCmd are present VSCmd takes precedence. |
| PitchCmd | sdddd | Commanded Pitch Angle s -(pitch down) +(pitch up) dddd tenths of degrees Note: Data field is empty when using VS guidance. If both VSCmd and PitchCmd are present VSCmd takes precedence. |
| Roll | sdddd | Current Roll Angle (measured by EFIS / AHRS) s -(rolling left) +(rolling right) dddd tenths of degrees |
| VS | sdddd | Current Vertical Speed (measured by EFIS / AHRS) s -(descending) +(climbing) dddd feet/min |
| Pitch | sdddd | Current Pitch Angle (measured by EFIS / AHRS) s -(pitching down) +(pitching up) dddd tenths of degrees |
| IAS | ddd | Current indicated airspeed (measured by EFIS / AHRS) ddd airspeed in whole knots |
| * | ending char | Sentence ending character |
| CS | Checksum | 2 ASCII character hexadecimal checksum (XOR of all characters between, but not including the $ and * characters) |
| <CR><LF> | Line termination | Line termination characters 0x0D and 0x0A |

The blue highlighted is data measured and sent by the EFIS / AHRS for informational purposes. Currently unused by AP

$PTFSB

The format of this message is:

$PTFSB,Mode*CS<CR><LF>

| Field | Data | Description |
| --- | --- | --- |
| $PTFSB | Identifier | Sentence identifier with $ starting character |
| Mode | d | Commanded AP Mode d “0” = Off “1” = AP standalone mode active “2” = Remote controlled mode active “6” = Remote controlled follow ARINC commands* “7” = Remote controlled HIGH GAIN mode active* |
| * | ending char | Sentence ending character |
| CS | Checksum | 2 ASCII character hexadecimal checksum (XOR of all characters between, but not including the $ and * characters) |
| <CR><LF> | Line termination | Line termination characters 0x0D and 0x0A |

Note:  ARINC command interface is included as appendix A of this document.

*Not currently implemented.

$PGRMZ (Not currently used by autopilot)

The format of this message is:

$PGRMZ,PressAlt,Baro*CS<CR><LF>

| Field | Data | Description |
| --- | --- | --- |
| $PTFSZ | Identifier | Sentence identifier with $ starting character |
| PressAlt | sddddd | Current pressure altitude (Measured by EFIS / AHRS) s -(negative altitude) +(positive altitude) ddddd altitude in whole feet |
| Baro | dddd | Current barometer setting dddd barometer setting in INHg with no decimal. Note: Barometer setting of 30.14 would be sent as 3014 |
| * | ending char | Sentence ending character |
| CS | Checksum | 2 ASCII character hexadecimal checksum (XOR of all characters between, but not including the $ and * characters) |
| <CR><LF> | Line termination | Line termination characters 0x0D and 0x0A |

The blue highlighted is data measured and sent by the EFIS / AHRS for informational purposes.

$PGASR

The format of this message is:

$PGASR,Mode,Trim,SpeedLimit,SelTrk,SelVS,Status*CS<CR><LF>

| Field | Data | Description |
| --- | --- | --- |
| $PGASR | Identifier | Sentence identifier with $ starting character |
| Mode | d | Current AP Mode d “0” = Off “1” = AP standalone mode active “2” = Remote controlled mode active “3” = Control Wheel Steering (CWS) mode active “6” = Remote controlled mode active using ARINC* “7” = Remote controlled HIGH GAIN mode active* |
| Trim | aa | Report need for trim adjustment aa “UP” = Need elevator up trim “DN” = Need elevator down trim |
| SpeedLimit | aaa | Report airspeed limiting active aaa “MIN” = Minimum airspeed limiting active “MAX” = Maximum airspeed limiting active |
| SelTrk | ddd | Manually selected autopilot track (AP standalone mode) ddd decimal degrees (000 to 359), magnetic Note: data field empty when not in AP standalone mode |
| SelVS | sdddd | Manullay selected vertical speed (AP standalone mode) s -(descend) +(climb) dddd feet/min (1 to 4 chars) Note: data field empty when not in AP standalone mode) |
| Status | ddd | Autopilot Status ddd sum of error bits, 0 means no error (1 to 3 chars) A non-zero status causes the G3X to display an autopilot fault annunciation |
| * | ending char | Sentence ending character |
| CS | Checksum | 2 ASCII character hexadecimal checksum (XOR of all characters between, but not including the $ and * characters) |
| <CR><LF> | Line termination | Line termination characters 0x0D and 0x0A |

*Not currently implemented

Note: When control wheel steering (CWS) is activated by the pilot, the autopilot should NOT send out an AP Mode of OFF (0) before sending the CWS (3) mode.  In other words, when the AP Disconnect/CWS switch is depressed, the autopilot should wait until it has determined which mode is being selected (Off or CWS) to notify the EFIS / AHRS of the new mode.

Note:  ARINC command interface is included as appendix A of this document.

Refer to NMEA 0183 documentation for detailed information on this protocol.

An empty NMEA field is signified by no characters being supplied between the field delimiting characters (normally commas). (e.g. ,, or ,*)

NMEA fields are not space or zero padded.  For example, the IAS field in the $PTFSV message may be 5, 45, 156, etc. but will not be 005 or 045.

ARINC 429 Interface

There shall be two wires connecting the EFIS to the AP, following the standards of the ARINC-429 bus, operating in Low Speed Mode. On this bus, the AP will be sensitive to the following labels ( label number in Octal, per ARINC documentation ).

- Label 121 Horizontal command

This label is used to command the autopilot to fly a continuously-varying desired angle of bank. The autopilot will limit the incoming command to no more than 30 degrees, and it will limit the effective rate-of-change of the command to no more than ten degrees per second.

- Label 104 Selected Vertical Speed command

This label contains a continuously-varying commanded vertical speed target for the autopilot to maintain or achieve. The command will be limited to +/- 3000 FPM internally to the AP. By manipulating the commanded vertical speed, the EFIS can implement any desired vertical mode, whether that be altitude-hold, vertical-speed hold, VNAV, or profile climbs and descents.

The AP contains a pre-set Minimum and Maximum indicated airspeed value, and should the aircraft ever be flying outside these limits, the AP will abide by the commanded vertical speed only to the extent that is possible without violating the pre-set airspeed limits.

- Combination of label 121 and 104

In order to enter and remain in the Remote mode, both labels 121 and 104 must be present and not flagged. If the autopilot is in Remote mode and one or both of the labels becomes missing, timed-out, or flagged, the AP will switch to “On” mode, capturing the existing direction of flight, and the existing altitude at the moment of loss of the information.

Once in the “On” mode, the user may, on the AP controls, select direction of flight and vertical speed. Once both labels are restored, the autopilot may be set to Remote mode either by the EFIS (using the $PTFSB message) or by pressing the “EFIS” button on the autopilot.

- Label 320 Magnetic Heading

ARINC Label 320 (Magnetic Heading) is used by the autopilot in case the GPS Ground Track portion of the $GPRMC message becomes missing or flagged, or should the ground speed be near zero, making a ground track an unreliable indication of direction of motion. The only time label 320 has meaning to the autopilot is when the aircraft is nearly stationary, or GPS information is lost. This allows the AP to fly a selected magnetic heading without any NMEA-based GPS information available.

- Label 324 Pitch Angle

Pitch Angle is used as a pitch attitude reference.

- Label 325 Roll Angle

Roll Angle is used as a bank angle reference and for bank angle limiting.

If the remote autopilot is a Sorcerer, further integration is possible.  The following commands will enable tighter interface even when the Sorcerer is being operated in both EFIS and AP modes.

- Label 101 Selected Heading

This allows the Sorcerer to follow a heading bug using the External DG mode (EXT button) on the Sorcerer.  This feature will not work if the Sorcerer is receiving Label 377 with equipment code of 0x25.  To follow a heading bug when using the Sorcerer in EFIS mode, send heading command under Label 121 Horizontal Command.

- Label 102 Selected Altitude

Sending Selected Altitude to the Sorcerer will enable the “auto nominate” feature, which automatically sets the selected altitude to agree with the EFIS.  This only works when the SEL button is pressed on the Sorcerer to initiate a climb/descent directly on the Sorcerer.

- Label 100 Selected Track

Selected Track is used to “auto nominate” the desired track when using NAV or REV modes (VOR / Loc / ILS)

- Label 204 Baro Altitude

Baro Altitude along with Baro Setting (Label 235) are both used to ensure that the Sorcerer altimeter is always in agreement with the EFIS altimeter.

- Label 235 Baro Setting

Baro Setting along with Baro Altitude (Label 204) are both used to ensure that the Sorcerer altimeter is always in agreement with the EFIS altimeter.

- Label 377 Equipment ID

This MUST be 0x25 for the Sorcerer to operate in EFIS mode.  Equipment ID must be sent at least every 2 seconds or the EFIS mode will time-out and the Sorcerer revert back to track / vertical speed mode.

The AP will receive ARINC A and B on pins 14 and 15 of the 25 pin connector

- Timing Requirements

RS-232 Messages input to the autopilot are $GPRMC and $ PTFSB.  For proper dynamic operation, the $GPRMC message is required at least once per second. The autopilot uses this information to maintain its sense of direction when it is engaged independently of the EFIS or operating stand-alone.  No other standard NMEA messages are required. If present they will be skipped.

Note: The $GPRMB message is used by many autopilots for implementation of the GPS Nav mode; however in the case of the dedicated EFIS-controlled AP the $GPRMB message will have no function and will be ignored.

The $PGASR Messages sent by the autopilot on the RS-232 output will be sent every 2 seconds, or sooner, if any component of the message has changed since its last transmission by the autopilot. If the AP’s RS232 output does not contain a message for 3 or more seconds, the external equipment may assume that there has been a malfunction within, or a loss of power to, the autopilot.

ARINC Label 121 (horizontal steering command) needs to be received by the autopilot at a nominal rate of 10 times per second. Correct dynamic response cannot be accomplished if the frequency is less than two times per second. A timeout error will occur if ½ second elapses between receptions of this label.

ARINC Label 104 (vertical speed command) needs to be received by the autopilot at a nominal rate of 10 times per second. Correct dynamic response cannot be accomplished if the frequency is less than two times per second. A timeout error will occur if ½ second elapses between receptions of this label.

ARINC Label 320 (Magnetic Heading) is optional. If label 320 is sent, it should be sent at a frequency of 5 or more times per second.

- Error Conditions

- Timeout of messages on the RS232 bus

$GPRMC is deemed to have timed out if not received for 3 seconds or more.

$PTFSB has no timeout and need not be sent continuously.

- Timeout of messages on the ARINC bus

Labels 121,104 and 320 are deemed to have timed out if not received for 500 Milliseconds or more. Each label has its own timer.

- Errors Internally detected by the autopilot include, but may not be limited to the following:

- Failure of self-test after power up will result in error code 1

- Failure of an autopilot gyro will result in error code 2, 4, or 8, for Azimuth, Roll, and Pitch respectively.

- Failure of an internal voltage reference or similar analog error will result in error code 16.

- Other possible error codes may be described in later revisions of this document.